CITY OF BUCHANAN

COUNTY OF BERRIEN, STATE OF MICHIGAN

ORDINANCE 2026.02/446

THE CITY OF BUCHANAN ORDAINS THAT CHAPTER 102 Utilities, ARTICLE II. DOWNTOWN DEVELOPMENT AUTHORITY of the City of Buchanan Code of Ordinance is hereby amended by adding the following:

ARTICLE V. - SEWAGE DISPOSAL

Sec. 102-156. - Definitions.

The following words, terms and phrases, when used in this article, shall have the meanings ascribed to them in this section, except where the context clearly indicates a different meaning:

BOD means biochemical oxygen demand or the quantity of oxygen utilized in the biochemical oxidation of organic matter under standard laboratory procedure in five days at 20 degrees Celsius, expressed in milligrams per liter.

Building drain means that part of the lowest horizontal piping of a drainage system which receives the discharge from soil, waste and other drainage pipes inside the walls of the building and conveys it to the building sewer, beginning five feet outside the inner face of the building wall.

Building sewer means the extension from the building drain to the public sewer or other place of disposal.

Commercial wastes means the liquid or water-carried wastes from commercial establishments or those concerns engaged in buying, selling, or exchanging goods and/or services.

Director means the superintendent of wastewater treatment plant of the city under the direction of the city manager, or his authorized deputy, agent or representative.

Garbage means the waste from the preparation, cooking and dispensing of food and from the handling, storage and sale of produce.

Industrial wastes means the liquid or water-carried wastes from industrial processes as distinct from sanitary sewage.

mg/I means milligrams per liter.

Natural outlet means any outlet into a watercourse, pond, ditch, lake or other body of surface water or groundwater.

pH means the logarithm of the reciprocal of the hydrogen ion concentration in grams per liter of solution.

Properly shredded garbage means garbage that has been shredded to such a degree that all particles shall be carried freely under the flow conditions normally prevailing in the public sanitary or combined sewer; with no particles larger than one-half inch in any dimension.

Public sewer means a sewer that is controlled by the city.

Sanitary sewage means the liquid or water-carried waste discharged from the sanitary conveniences of dwellings (including apartment houses and hotels), office buildings, and commercial establishments where the public is served.

Sanitary sewer means a sewer to which stormwaters are not intentionally admitted.

Sewage means any liquid or water-carried waste from residences, business buildings, institutions, laboratories, industrial and commercial establishments, together with stormwaters as may be present.

Sewage disposal system or system means the complete sewage disposal system of the city, and shall be construed to include all plants, works, collecting mains, instrumentalities and properties, now or hereinafter existing, used or useful in connection with the collecting, pumping, disposal and treatment of sanitary sewage and industrial wastes, as now or hereafter added to, expanded or improved.

Sewage treatment plant means any arrangement of devices and structures used for treating sewage.

Sewer means a pipe or conduit for carrying sewage.

Storm drain or storm sewer means any drain or sewer, either natural or artificial, which is intended expressly for the conveyance of stormwater and uncontaminated industrial wastes.

Stormwater means that part of precipitation which reaches the sewers as runoff from the natural land surface, building roofs, or pavements or as groundwater infiltration.

Suspended solids means solids that either float on the surface of, or are in suspension in water, sewage or other liquids, and which are removable by laboratory filtering in accordance with standard laboratory procedure.

Wastewater means the same as or is equivalent to "sanitary sewage."

Watercourse means an open, natural channel in which a flow of water occurs either continuously or intermittently.

(Comp. Ords. Rev. 1991, § 25.002)

Cross reference- Definitions generally, § 1-2. Sec. 102-157. - Organization.

- The sewerage system of the city, including all property and employees in connection therewith, shall be known as the Buchanan sewerage department, and shall be under the direction of the city manager.

- Authority for construction, repair and maintenance of the sewage disposal system may be delegated by the city manager to the superintendent of environmental services.

The city treasurer shall collect all money due to the sewage department and shall maintain an accurate separate account of the same.

(Comp. Ords. Rev. 1991, § 25.001) Sec. 102-158. - Management of system.

- The sewers of the department are under the exclusive control of the commission and no person

other than agents or employees of the department shall disturb, tap, change, obstruct or interfere with them in any way.

- Extensions of, or changes in the sewers shall be made only by the direction of the commission. Petition for the extension of old or construction of new sewers shall be addressed to the commission who will thereupon consider same and advise the petitioners of their decision. If the petition be granted, the commission shall stipulate the proportions of the cost to be borne by the petitioners and by the department.

- Any person installing sewers at their own expense shall first submit plans and specifications for such work to the manager for state approval. After such plans and specifications have been approved the work shall be done under the supervision of the manager, who shall require such test made as he considers necessary, and shall accept the installation for the city before any sewer service pipe shall be placed in service. All inspection cost for such installation shall be paid by the party installing this sewer. The provisions of this subsection shall also apply to any installations of sewers outside the corporate limits of the city where permission may be granted by the commission to connect to the city sewers.

(Comp. Ords. Rev. 1991, § 25.003)

Sec. 102-159. - Scope of regulations.

The standards and regulations established in this article are deemed to be the absolute minimum consistent with the preservation of the public health and safety and to fulfill the obligations of the city with respect to state and federal law and all rules and regulations adopted pursuant thereto. The discharge into any sewer in the city of any substance which exceeds the limitations contained in this article, or in any manner fails to conform, is hereby declared to be a public nuisance, and a violation of this Code.

(Comp. Ords. Rev. 1991, § 25.004)

Sec. 102-160. - Powers and authority of inspectors.

The director and other duly authorized employees of the city bearing proper credentials and identification shall be permitted to enter upon all properties for the purpose of inspection, observation, measurement, sampling and testing in accordance with the provisions of this article. Any person who

applies for and/or receives sewer services from the city under this article shall be deemed to have consented to inspections pursuant to this section, including entrance upon that person's property at reasonable times to make inspections under this article.

(Comp. Ords. Rev. 1991, § 25.016) Sec. 102-161. - Reserved.

Sec. 102-162. - Notice of violation.

Any person found to be violating any provision of this article shall be served by the city with written notice stating the nature of the violation and providing a reasonable time limit for the satisfactory correction thereof. The offender shall, within or at the expiration of the period of time stated in such notice permanently cease all violations. Any person who shall continue any violation beyond the time limit provided shall be guilty of a violation of this Code.

(Comp. Ords. Rev. 1991, § 25.022)

Sec. 102-163. - Penalty.

Any person violating any of the provisions of this article shall be guilty of a misdemeanor, and upon conviction thereof before any court of competent jurisdiction shall be fined as prescribed in section 1-15 of this Code.

(Comp. Ords. Rev. 1991, § 25.023)

Sec. 102-164. - Protection of system from damage.

No unauthorized person shall maliciously, willfully or negligently break, damage, destroy, uncover, deface or tamper with any structure, appurtenance, or equipment which is a part of the municipal sewage works.

(Comp. Ords. Rev. 1991, § 25.015)

Sec. 102-165. -Application for service connection.

- Before any connection shall be made to any sewer, application for same shall be made in writing to the department by the owner of the premises to be served, or by his or her authorized agent. A condition to the granting of a service connection shall be an agreement by the owner to abide by all of the rules and regulations of the department in all respects but more especially with those respecting the responsibility for payment for sewage disposal service.

- That part of the sewer service pipe on private property shall be inspected by the department prior to covering the pipe. The fee for such inspection shall be set by resolution of the city commission.

- Whenever the ownership of a premises changes, the use by the new owner or his or her agent or tenant of any of the facilities of the department shall be prima facie evidence that the new owner agrees to abide by the rules and regulations of the department and requirements of this article, but more especially with those respecting the responsibility for the payment for sewage disposal service.

(Comp. Ords. Rev. 1991, § 25.018) Sec. 102-166. - Payment for service.

- Bills for sewage disposal service shall be levied on each lot or parcel of land, building or premises having access to any sewer, or discharging sewage or waste into the sewage system, either directly or indirectly. These sewage disposal bills may be rendered with the water bills and made payable at the same time and under the same conditions as for the water bills.

- Billing for water consumption as provided for in another ordinance, and for sewage disposal as provided by this article shall not be severable but shall be paid as one bill.

- The rates charged for sewage disposal service shall be established by resolution of the city commission. The sewer rates shall be independent of the water rates.

(Comp. Ords. Rev. 1991, § 25.019)

Sec. 102-167. - Use of sewage disposal system.

No person shall discharge or cause to be discharged any material or waste into the sewage disposal system except in compliance with section 102-211.

(Comp. Ords. Rev. 1991, § 25.005)

Sec. 102-168. - Interceptors.

- Grease, oil and sand interceptors shall be provided for the handling of liquid wastes containing grease or oil in excessive amounts, or any flammable wastes, sand and other harmful ingredients. All interceptors shall be of a type and capacity approved by the director, and shall be located so as to be readily and easily accessible for cleaning and inspection. Grease and oil interceptors shall be constructed of impervious materials capable of withstanding abrupt and extreme changes in temperatures. They shall be of substantial construction, watertight and equipped with easily removable covers which, when bolted in place, shall be gastight and watertight.

- Where installed, all grease, oil and sand interceptors shall be maintained by the owner, at his expense, in continuously efficient operation at all times.

(Comp. Ords. Rev. 1991, §§ 25.006, 25.007)

Secs. 102-169-102-173. - Reserved.

Sec. 102-174. - Storm sewer system.

No person shall discharge or cause to be discharged into any storm sewer or natural or artificial watercourse, waters or wastes other than stormwater or uncontaminated industrial wastes as heretofore defined, except upon special agreement or arrangement with the city, which may seek review by the appropriate state or local agency.

(Comp. Ords. Rev. 1991, § 25.014)

Sec. 102-175. - Sewer and drain connections required.

The owner or occupant of any property situated within the city upon which is located a structure or structures in which water is used or available for household, commercial, industrial or other purposes shall, at his own expense, cause such property to be connected to an available public sanitary sewage collection facility. Such owner or occupant shall also be required to install suitable toilet facilities within such structures.

(Comp. Ords. Rev. 1991, § 25.017)

Sec. 102-176. - Disposal of waste from private systems at sewage treatment plant.

Any water or waste discharged into the sewage disposal system shall comply with the requirements of section 102-211.

(Comp. Ords. Rev. 1991, § 25.020)

Sec. 102-177. - Reserved.  Sewer Laterals and Appurtenances

(a)  The property owner shall maintain, at its, his or her expense, the sewer lateral to ensure continuous flow of sewage from the structure to the sewer main, local collector, trunk or interceptor sewer, and shall be responsible for all maintenance and/or replacement of lateral sections existing between the structure and the public sewer line.  The property owner is responsible for its lateral from its structure to the sewer main, local collector, trunk or interceptor sewer even if the lateral is located underneath a city street or right-of-way.  In the event that a sanitary service pipe is in need of repair or is otherwise nonconforming with the law or any rules and regulations adopted by the city or under this article, and the property owner fails to correct the nonconformity after being provided notice, and in the sole judgment of the city, it is in the interest of public health, safety or welfare to initiate and manage the correction, the city may cause the same to be connected, repaired or replaced and collect the cost thereof, plus a five percent administrative fee, up to a maximum of $_______.00, from the property owner by an action at law, or may charge the same as a special assessment against the lands and premises served by the water service pipe and collected in the same manner as an ad valorem property tax, charged upon and collected with the next practical property tax roll immediately following the correction. Performance of such work by the city shall not relieve any person from the penalties otherwise prescribed by law or as set forth in this article.

(b)   Whenever a sewer lateral is to be extended into the premises, the plumber or contractor may obtain location measurements from the city. However, the city does not assume responsibility for the accuracy of such location measurements.  Existing sewer service laterals may be used in connection with new buildings only if they are found, on examination and test by the director to meet all requirements of this chapter and other applicable laws and regulations. If an inspection by the city reveals that a connection may create a health or environmental hazard, nuisance, or is otherwise inconsistent with the purposes and requirements of this chapter, the sewer service lateral shall be reconstructed or repaired at the owner's expense.

(c)  There shall not be more than one structure served by a single sewer lateral connection, except with approval of the city manager.

(d)  When a structure is to be demolished, satisfactory arrangements shall be made by the property owner with the city to disconnect and seal the sewer lateral at the property line or at the point designated by the city at the property owner’s sole expense. The lateral disconnection shall be inspected by the city prior to sealing. Failure to make arrangements for inspection and the proper termination of the connection shall cause the City Manager to order excavation of the lateral for the required inspection with all associated costs to be assessed against the property owner.

(e)   Roof drains shall not be connected to the system. The City Manager shall order the immediate disconnection of all such connections. The owner of the premises shall be responsible for any and all costs associated with these disconnections and all costs including, but not limited to, legal and inspection service required to enforce provisions of this chapter. Each day the owner fails to comply with such order shall constitute a separate violation of this section.

(f)   Footing drains shall not be connected to the system on any structure. The City Manager shall order the immediate disconnection of such connections. The owner of the premises shall be responsible for any and all costs associated with these disconnections and all costs including, but not limited to, legal and inspection service required to enforce provisions of this chapter. Each day the owner fails to comply with such order shall constitute a separate violation of this section.

(g) Basement waterproofing systems shall not be connected into the sewer system or discharged in such a manner as to cause a public or private nuisance. Prior to installation of a basement waterproofing system, the property owner, or the owner’s contractor, must obtain a permit from the city. The waterproofing system must be inspected and approved by the city prior to putting the system into operation. The city manager may order the owner to discontinue the discharge of water from a basement waterproofing system. Each day the owner fails to comply with such order shall constitute a separate violation of this section.

Sec. 102-178. - User charges.

- Established. Pursuant to Section 601-34, Industrial Waste Treatment, Title 18, Chapter V, Federal Register, Volume 35, No. 128, Thursday, July 2, 1970, user charges for operation and maintenance of publicly owned treatment works are herein established. The operation and maintenance costs shall include debt retirement.

- Implementation. Implementation of user charge system assures that each recipient of waste treatment services will pay its proportionate share of the costs of operation and maintenance of treatment works provided by the city.

(c)  Definition of industrial user. The industrial user shall mean any industry identified in the Standard Industrial Classification Manual, Bureau of the Budget, 1967, as amended and supplemented, under the category Division D-Manufacturing, and such other wastes as deemed appropriate by the city commission. A user listed in the said division may be excluded if it is determined that it will introduce primarily segregated domestic wastes or wastes from sanitary conveniences.

(d)	Factors influencing share. An industrial user's share shall be based on all factors which significantly influence the cost of the treatment works. Factors such as strength, volume and delivery flow rate characteristics shall be considered and included to ensure a proportional distribution of the operation and maintenance costs allocable to all industrial users of the treatment works.

(e)	Adjustment of share. When a substantial change in the strength, volume or delivery flow rate characteristics occurs in the wastewater introduced to the treatment works by an industrial, institutional or commercial user, such user's share shall be adjusted accordingly to generate revenue adequate to treat the wastewater contributed by such user.

(f)	Formula for charges. Users of the sewage disposal system shall be charged a wastewater collection and treatment rate based on the quantity formula, subject to surcharges for higher strength wastewater in accordance with the amended rate ordinance.

(g)	Estimates. Any residential, industrial, commercial or institutional user subject to a user charge shall furnish, as and when requested by the city commission, an estimate of the volume discharged into the wastewater treatment systems; and an estimate of the representative value of the strength of the waste, BOD and suspended solids discharged, whether or not such amounts are in excess of the permitted amount. All measurements, tests and analyses of the characteristics of such wastewater shall be determined in accordance with this chapter or by other methods generally accepted under the established sanitary engineering practices and approved by the superintendent. The reports submitted shall be subject to certification by the superintendent, but may serve as the basis for billing with all necessary adjustments in the amounts to be made after verification. If an analysis and volume of the waste is not furnished to the city commission in the specified time, the charges shall be based upon estimates made by the superintendent, and approved by the city commission. The superintendent shall have the right at any reasonable time to enter upon the land of the user to set up such equipment as is necessary to verify the reports submitted. In cases where measurements are difficult to make, or the industrial waste composition changes frequently, or representative samples are difficult to get or where other methods of measurements are necessitated for other sound engineering reasons as determined by the superintendent, the superintendent shall have the authority to use such other basis for determining said charges as shall be reliably indicative of volume and BOD and SS strength of the particular industrial waste, such as, but not limited to, water purchase or usage, character of products, comparisons between the subject data and collection data from like

persons. All billings for said charges shall be payable within ten days after mailing of billings. All payments made, based upon the reports submitted, shall become final unless verification is made and notice given by the superintendent of necessary adjustments within one year of the payment. Underpayment of charges based on errors in users' reports and estimates shall be billed forthwith on ascertainment thereof.  Overpayment of charges arising from any cause shall first be applied to unpaid billings and then the excess forthwith refunded.

(h)	Billing estimates. The superintendent shall have the authority to estimate the volume and strength of the industrial waste in the event the user subject to the rate and charges fails to file a timely report as provided in this section. The estimates shall be based upon analyses and volumes of a similar installation or the amount of water supplied to the premises by the water utility, the amount of water supplied to the premises by any private sources of water or volume and analysis as determined by measurements and samples taken by the superintendent or an estimate determined by the superintendent by any combination of the foregoing or by any other equitable method. Failure to file written objection to such estimates within ten days after the mailing of written notice thereof of the user liable therefor shall constitute a waiver of any right to object or appeal the estimates made by the superintendent pursuant to this section.

(i)	Appeals to the city commission. Any user may appeal the charges assessed against him to the city commission and shall have a hearing upon the following conditions:

  - That the user submits estimates or authorizes the superintendent to make such estimates.

  - That the user has good cause to believe that the charges assessed are in error.

  - That notice in writing has been given to the city commission within 180 days of receipt of the charges in question.

The city commission shall notify the user making the appeal of the time and place when his appeal will be heard. Upon evidence sufficient to the city commission submitted at the hearing that the charges are in error, the city commission shall make adjustments in the charges. Adjustments may be in the form of a refund or a credit against subsequent assessments of the charges provided for in this article.

(j) Rate review. The city commission shall cause an annual report to be made concerning the effect of the user charge system on the various classes of users. Such report shall include the amount, character and strength of the wastes discharged into the sewerage system and accounting of the revenues produced by the various classes of users. The city commission may appoint a committee to aid and assist in the preparation of the report and the committee membership may include representatives of various classes of users and other persons professionally qualified to work on the committee as determined by the city commission. If the revenues are shown in the report to be inequitable, the city commission will review the user charges and cause the necessary adjustments to be made in the rate structure.

(Comp. Ords. Rev. 1991, § 25.029)

Secs. 102-179-102-200. - Reserved.

This Ordinance shall become effective fifteen (15) days after its adoption and publication as required by Section 7.4 of the City Charter.

MOTION MADE, PASSED, AND ADOPTED BY THE CITY COMMISSION OF THE CITY OF BUCHANAN, BERRIEN COUNTY, MICHIGAN ON THE 	TH DAY OF 		 2026 AND IT WAS PUBLISHED IN THE BERRIEN COUNTY RECORD NEWSPAPER ON THE 	ND DAY OF

By

Mark Weedon, Mayor

Kalla Langston-Weiss, City Clerk

CERTIFICATION, I herby certify that the above is a true and complete copy of an ordinance adopted by the City Commission of the City of Buchanan, County of Berrien, State of Michigan, at a regular meeting held on 	day of 		, 2026, and that said meeting was conducted and public notice of said meeting was given pursuant to and in full compliance with Act No. 267, Public Acts of Michigan, 1976 as required by said act.

Kalla Langston-Weiss, City Clerk